# Introduction

Endorheic watersheds in Africa present distinct challenges for remote sensing. Their shallow, turbid waters confound optical classification. Their margins shift seasonally and inter-annually across kilometres of flat terrain. Their ecological and economic significance is mediated by social systems that satellites cannot observe. Remote sensing provides spatial data essential to wetland monitoring (Eva & Lambin, 2000; Ozesmi & Bauer, 2002; Philippe & Karume, 2019), yet it captures neither the adaptive strategies of local resource users nor the political ecology that governs access to fluctuating resources (Demichelis et al., 2020; Sulieman & Ahmed, 2013; Yiran et al., 2012).

Technical limitations compound this problem. Inland waters differ fundamentally from the terrestrial surfaces for which most satellite sensors were designed. Coloured dissolved organic matter (CDOM), suspended sediment, and phytoplankton blooms alter the spectral properties of water in ways that atmospheric correction algorithms handle poorly (Matthews, 2011; Ogashawara et al., 2017). Higher-resolution sensors introduce their own constraints in spectral coverage, radiometric sensitivity, and temporal frequency (Kutser, 2012; Palmer et al., 2015). The challenge is not merely technical. It is that remote sensing alone produces maps that are technically defensible but ecologically and socially incomplete.

This study addresses both limitations simultaneously. We combine multi-sensor remote sensing with sustained ethnographic fieldwork to map inundation dynamics in the Lake Chilwa Basin, one of Africa’s most productive and volatile endorheic systems. The remote sensing component evaluates optical water-extraction indices against SAR-derived water maps to quantify what each sensor family detects and what it misses. The ethnographic component, conducted over 18 months across three districts, validates the satellite-derived classifications and reveals the socio-ecological structures that determine how the lake’s resources are governed, contested, and used.

## The Problem with Optical Indices in Inland Wetlands

Water-extraction indices derived from multispectral imagery are the standard tools for mapping surface water extent. Each exploits a different combination of spectral bands to distinguish water from land, but each fails under specific conditions that Lake Chilwa routinely presents.

The Normalized Difference Water Index (NDWI; McFeeters, 1996) uses the ratio of green to near-infrared reflectance to delineate open water. It performs well where water is deep and clear (Ji et al., 2009) but struggles in shallow, turbid conditions because suspended sediment raises NIR reflectance, compressing index values toward zero (W. Li et al., 2013; H. Xu, 2006). In vegetated wetlands, canopy reflectance dominates both bands and the index cannot distinguish water beneath emergent vegetation from surrounding marshland (Ozesmi & Bauer, 2002). Endorheic lakes with fluctuating shorelines present mixed pixels at the land-water boundary where NDWI thresholds systematically misclassify wet soil as dry land (Deus & Gloaguen, 2013).

The Modified NDWI (MNDWI; Xu, 2006) substitutes shortwave infrared for NIR, suppressing the confusion between water and built-up or bare-soil surfaces that afflicts NDWI. It performs better in turbid and sediment-laden water (Acharya et al., 2018; W. Li et al., 2013) and is the strongest single optical index for the conditions this study addresses. It still fails beneath emergent vegetation, where macrophyte reflectance drives values negative even where standing water persists (Amani et al., 2020). In saline and endorheic systems, dissolved salts and algal blooms shift MNDWI thresholds unpredictably, requiring adaptive or Otsu-based methods (Pekel et al., 2016). Heimhuber et al. (2018) found that MNDWI required SAR fusion to map water extent beneath vegetation in the Murray-Darling system.

The Automated Water Extraction Index (AWEIsh; Feyisa et al., 2014) combines five bands to suppress shadow and dark-surface commission errors. Its multi-band formulation captures contrasts that two-band indices cannot resolve, but subpixel mixing of emergent vegetation and water still reduces values below detection thresholds (Ji et al., 2015). Fisher et al. (2016) found that no single index, AWEIsh included, consistently mapped shallow or seasonal water bodies in a global comparison. In endorheic systems, AWEIsh can misclassify exposed lakebed and brine as water (Pekel et al., 2016).

The Water Ratio Index (WRI; Shen and Li, 2010) uses the ratio of visible to infrared bands, yielding values above 1.0 for water. It suppresses cloud and shadow noise effectively (J. Li et al., 2023) and has shown strong performance in bare-soil landscapes in Ethiopia (Demelash et al., 2025). But where water, vegetation, and sediment share a pixel, the visible-band numerator inflates from non-water reflectance. The red band is particularly vulnerable to suspended sediment, which raises its reflectance sharply in turbid conditions. WRI has not been applied to endorheic or fluctuating lake systems.

The Normalized Difference Pond Index (NDPI; Lacaux et al., 2007) was designed in and for semi-arid Africa, using SPOT-5 imagery to classify temporary ponds in Senegal’s Ferlo region. It exploits the SWIR absorption of water against green vegetation reflectance, making it sensitive to the water-vegetation boundary where other indices fail. Campos et al. (2012) confirmed that SWIR-based indices outperform NIR-based ones for ephemeral ponds in the Sahel. NDPI is the algebraic inverse of MNDWI, carrying identical spectral information with reversed sign convention. It performs best on small, shallow, vegetated water bodies and loses sensitivity in deep or highly turbid water.

The pattern across all five indices is consistent: each occupies a spectral niche but fails at the boundaries that define Lake Chilwa’s hydrology. Shallow turbid water, vegetated wetland margins, fluctuating shorelines, and mixed pixels at the land-water transition are precisely the conditions where optical classification underperforms. These are also the zones of greatest ecological and socio-economic significance.

## SAR as a Complement to Optical Methods

Synthetic aperture radar addresses the specific failures of optical indices in inland wetlands. SAR operates at microwave frequencies, independent of cloud cover and solar illumination, two properties that make it indispensable for monitoring tropical wetlands where persistent cloud obscures optical sensors for months during the critical wet season (Mahdavi et al., 2018).

Smooth open water produces near-specular reflection, returning very little backscatter to the sensor. C-band SAR typically records open water at -20 to -30 dB, well below surrounding land surfaces, enabling detection through simple thresholding (Martinis et al., 2015). Where vegetation stands in water, the signal follows a double-bounce pathway, reflecting from the water surface and then from vertical plant structures. This produces backscatter paradoxically higher than from the same vegetation when dry, because the smooth water surface enhances the coherent specular return (Hess et al., 1995; Tsyganskaya et al., 2018). The contrast between double-bounce returns from flooded vegetation and volume scattering from dry canopies is the physical basis for detecting sub-canopy inundation.

Sentinel-1, operational since 2014, provides C-band imagery at 10 m resolution with 6 to 12 day revisit. Its dual-polarisation mode (VV/VH) has been applied to wetland inundation mapping across diverse environments: the St. Lucia wetlands in South Africa (Clement et al., 2018), the Amazon lowlands (Hardy et al., 2019), the Okavango Delta (Luca et al., 2025), and seasonal lake cycles on the Tibetan Plateau (Zhang et al., 2019). Multi-temporal approaches exploit wet-dry season contrast to characterise hydroperiod without ground data.

C-band has limitations. Speckle noise degrades classification accuracy. Wind roughens water surfaces, raising backscatter above detection thresholds and causing false negatives (Martinis et al., 2015). Most critically, C-band’s 5.6 cm wavelength cannot penetrate dense emergent vegetation such as Typha and Phragmites. Clement et al. (2018) reported poor classification in heavily vegetated zones. L-band SAR (approximately 23 cm wavelength), carried by ALOS PALSAR and the forthcoming NISAR mission, penetrates canopies far more effectively. Hess et al. (2003) showed that L-band detected sub-canopy flooding at 50 cm depth in Amazonian floodplains, versus 80 cm for C-band.

SAR interferometry offers additional capability. InSAR measures phase differences between acquisitions to detect centimetre-scale vertical displacement. In wetlands, the double-bounce mechanism preserves coherence, enabling water level change detection at resolutions exceeding gauge networks (Hong & Wdowinski, 2017; Wdowinski et al., 2008). Kim et al. (2021) mapped water level gradients across Tonle Sap at sub-monthly intervals. InSAR water level retrieval has not been attempted for African endorheic systems.

Fusing SAR and optical data exploits their complementarity. Pixel-level stacking, decision-level fusion, and machine learning classifiers trained on combined feature spaces consistently outperform single-source models, with reported accuracies of 85 to 95 percent for wetland classes (Amani et al., 2019; Whyte et al., 2018). Xu et al. (2025) mapped surface water dynamics across East Africa at 10 m resolution by integrating Sentinel-1 and Sentinel-2 time series. These studies confirm that multi-sensor integration captures dynamics that either sensor family misses alone.

## Study Area

Lake Chilwa Basin is one of Africa’s most productive endorheic ecosystems. Located in southern Malawi, this shallow terminal basin supports one of the continent’s most densely populated regions (500 km-1) and exhibits remarkable ecological productivity during wet periods (Njaya et al., 2011).

The basin’s hydrology is driven by unimodal rainfall (November to April) and sporadic chiperone rains (May to August). Annual fluctuations follow precipitation patterns closely (Ngongondo et al., 2011; Nicholson et al., 2014). Longer-term cycles of approximately 15 to 20 years produce dramatic lake recessions and varying degrees of complete desiccation (Kambombe et al., 2021). Historical records document major recessions in 1879, 1900, 1914-15, 1922, 1931-32, 1934, 1954, 1960-61, 1967, 1973, 1995, and 2012.

During recession, aquatic species take refuge in residual swamps dominated by salt-hardy Typha domingensis Pers. (Howard-Williams & Gaudet, 1985; Howard-Williams & Walker, 1974). Refilling initiates complex succession dynamics, with emergent food webs driven by detritus and bacterial processes in alkaline, nutrient-rich sediments (Furse et al., 1979; Kalk & Schulten-Senden, 1977). Peak productivity reaches 159 kg ha-1 (1979) and 113 kg ha-1 (1990), surpassing Lake Malawi (40 kg ha-1), Lake Tanganyika (90 kg ha-1), and Lake Victoria (116 kg ha-1) (Njaya et al., 2011; Vanden Bossche & Bernacsek, 1990). This boom-and-bust pattern sustains permanent lakeshore communities and seasonal migrant fishing populations.

## Research Objectives

This study presents an integrated remote sensing and participatory mapping approach to characterise lacustrine transgression-regression dynamics and associated socio-ecological patterns in the Lake Chilwa Basin. The objectives are:

- Evaluate five optical water-extraction indices (NDWI, MNDWI, AWEIsh, WRI, NDPI) and Sentinel-1 C-band SAR for mapping inundation extent across the basin’s full range of conditions, from deep open water through shallow turbid zones to vegetated marshland.

- Integrate SAR and optical time series using spectral mixture analysis to quantify hydroperiod fluctuations and littoral zone dynamics across multiple recession-refilling cycles.

- Validate satellite-derived classifications against ethnographic data collected through key informant interviews, focus group discussions, and participatory mapping with migrant fishing communities.

- Document the spatiotemporal structure of fishing regulations, population movements, and resource use patterns that formal monitoring frameworks have not captured.

Acharya, T. D., Subedi, A., & Lee, D. H. (2018). Evaluation of water indices for surface water extraction in a Landsat 8 scene of Nepal. Sensors, 18(8), 2580.

Amani, M. et al. (2019). Wetland mapping using multi-source and multi-temporal satellite imagery and environmental data. GIScience and Remote Sensing, 56(7), 1023–1045.

Amani, M. et al. (2020). Canadian wetland inventory using Google Earth Engine: The first map and preliminary results. Remote Sensing, 12, 1190.

Campos, J. C., Sillero, N., & Brito, J. C. (2012). Normalized difference water indexes have dissimilar performances in detecting seasonal and permanent water in the Sahara-Sahel transition zone. Journal of Hydrology, 464–465, 438–446.

Clement, M. A. et al. (2018). Multi-temporal synthetic aperture radar flood mapping using change detection. Journal of Flood Risk Management, 11(2), 152–168.

Demelash, T. et al. (2025). Evaluation of water extraction indices for mapping surface water dynamics. H2Open Journal, 8(5), 402.

Demichelis, C., Oszwald, J., Gasquet-Blanchard, C., Narat, V., Bokika, J.-C., Pennec, F., & Giles-Vernick, T. (2020). Multidimensional analysis of landscape dynamics in a central african forest-savannah mosaic. African Journal of Ecology, 58(4), 692708.

Deus, D., & Gloaguen, R. (2013). Remote sensing analysis of lake dynamics in semi-arid regions. Remote Sensing, 5, 5765–5781.

Eva, H., & Lambin, E. F. (2000). Fires and land-cover change in the tropics: A remote sensing analysis at the landscape scale. Journal of Biogeography, 27(3), 765776.

Fisher, A., Flood, N., & Danaher, T. (2016). Comparing Landsat water index methods for automated water classification in eastern Australia. Remote Sensing of Environment, 175, 167–177.

Furse, M., Morgan, P., & Kalk, M. (1979). The fisheries of lake chilwa (p. 209229). Springer.

Hardy, A. et al. (2019). Automatic detection of open and vegetated water bodies using Sentinel-1 to map African malaria vector mosquito breeding habitats. Remote Sensing, 11(6), 720.

Heimhuber, V. et al. (2018). Modeling 25 years of spatio-temporal surface water and inundation dynamics on large river basin scale using time series of Earth observation data. Remote Sensing of Environment, 209, 27–42.

Hess, L. L. et al. (1995). Delineation of inundated area and vegetation along the Amazon floodplain with the SIR-C synthetic aperture radar. IEEE Transactions on Geoscience and Remote Sensing, 33(4), 896–904.

Hess, L. L. et al. (2003). Dual-season mapping of wetland inundation and vegetation for the central Amazon basin. Remote Sensing of Environment, 87, 404–428.

Hong, S.-H., & Wdowinski, S. (2017). Double-bounce component in cross-polarimetric SAR from a new scattering target decomposition. IEEE Geoscience and Remote Sensing Letters, 14(8), 1370–1374.

Howard-Williams, C., & Gaudet, J. J. (1985). The structure and functioning of african swamps (P. Denny, Ed.; p. 153175). Springer Netherlands. https://doi.org/10.1007/978-94-009-5504-2_6

Howard-Williams, C., & Walker, B. (1974). The vegetation of a tropical african lake: Classification and ordination of the vegetation of lake chilwa (malawi). The Journal of Ecology, 831854.

Ji, L. et al. (2015). Improving the accuracy of the water surface cover type in the 30 m FROM-GLC product. International Journal of Applied Earth Observation and Geoinformation, 41, 109–117.

Ji, L., Zhang, L., & Wylie, B. (2009). Analysis of dynamic thresholds for the normalized difference water index. Photogrammetric Engineering and Remote Sensing, 75(11), 1307–1317.

Kalk, M., & Schulten-Senden, C. (1977). Zooplankton in a tropical endorheic lake (lake chilwa, malawi) during drying and recovery phases. Journal of the Limnological Society of Southern Africa, 3(1), 17.

Kambombe, O., Ngongondo, C., Eneya, L., Monjerezi, M., & Boyce, C. (2021). Spatio-temporal analysis of droughts in the lake chilwa basin, malawi. Theoretical and Applied Climatology, 144, 12191231.

Kim, S. et al. (2021). Monitoring Tonle Sap water level changes using Sentinel-1 SAR data. Remote Sensing of Environment, 252, 112150.

Kutser, T. (2012). The possibility of using the landsat image archive for monitoring long time trends in coloured dissolved organic matter concentration in lake waters. Remote Sensing of Environment, 123, 334338.

Li, J. et al. (2023). Surface water mapping from remote sensing images using water indices and machine learning. Remote Sensing, 15(6), 1678.

Li, W. et al. (2013). A comparison of land surface water mapping using the normalized difference water index from TM, ETM+ and ALI. Remote Sensing, 5(11), 5530–5549.

Luca, G. et al. (2025). Mapping seasonal flooding in the Okavango Delta using Sentinel-1 time series. Remote Sensing of Environment, 310, 114474.

Mahdavi, S. et al. (2018). Remote sensing for wetland classification: A comprehensive review. Remote Sensing of Environment, 206, 1–21.

Martinis, S. et al. (2015). Towards operational near real-time flood detection using a split-based automatic thresholding procedure on high resolution TerraSAR-X data. Natural Hazards and Earth System Sciences, 15(5), 1069–1085.

Matthews, M. W. (2011). A current review of empirical procedures of remote sensing in inland and near-coastal transitional waters. International Journal of Remote Sensing, 32(21), 68556899.

Ngongondo, C., Xu, C.-Y., Gottschalk, L., & Alemaw, B. (2011). Evaluation of spatial and temporal characteristics of rainfall in malawi: A case of data scarce region. Theoretical and Applied Climatology, 106, 7993.

Nicholson, S., Klotter, D., & Chavula, G. (2014). A detailed rainfall climatology for malawi, southern africa. International Journal of Climatology, 34(2), 315325.

Njaya, F., Snyder, K. A., Jamu, D., Wilson, J., Howard-Williams, C., Allison, E. H., & Andrew, N. L. (2011). The natural history and fisheries ecology of lake chilwa, southern malawi. Journal of Great Lakes Research, 37, 1525.

Ogashawara, I., Mishra, D. R., & Gitelson, A. A. (2017). Remote sensing of inland waters: Background and current state-of-the-art (p. 124). Elsevier.

Ozesmi, S. L., & Bauer, M. E. (2002). Satellite remote sensing of wetlands. Wetlands Ecology and Management, 10, 381402.

Palmer, S. C., Kutser, T., & Hunter, P. D. (2015). Remote sensing of inland waters: Challenges, progress and future directions.

Pekel, J.-F. et al. (2016). High-resolution mapping of global surface water and its long-term changes. Nature, 540, 418–422.

Philippe, M. T., & Karume, K. (2019). Assessing forest cover change and deforestation hot-spots in the north kivu province, DR-congo using remote sensing and GIS. American Journal of Geographic Information System [En Línea], 8(2), 3954.

Sulieman, H. M., & Ahmed, A. G. M. (2013). Monitoring changes in pastoral resources in eastern sudan: A synthesis of remote sensing and local knowledge. Pastoralism: Research, Policy and Practice, 3(1), 116.

Tsyganskaya, V. et al. (2018). SAR-based detection of flooded vegetation. International Journal of Applied Earth Observation and Geoinformation, 73, 205–218.

Vanden Bossche, J.-P., & Bernacsek, G. M. (1990). Source book for the inland fishery resources of africa (Vol. 1). Food & Agriculture Org.

Wdowinski, S. et al. (2008). Space-based detection of wetlands’ surface water level changes from L-band SAR interferometry. Remote Sensing of Environment, 112, 681–696.

Whyte, A. et al. (2018). A new synergistic approach for monitoring wetlands using Sentinels-1 and 2 data with object-based machine learning. Environmental Modelling and Software, 104, 40–54.

Xu, H. (2006). Modification of normalised difference water index (NDWI) to enhance open water features in remotely sensed imagery. International Journal of Remote Sensing, 27(14), 3025–3033.

Xu, Y. et al. (2025). Mapping surface water dynamics across East Africa at 10 m resolution. ISPRS Journal of Photogrammetry and Remote Sensing, 215, 101–118.

Yiran, G., Kusimi, J., & Kufogbe, S. (2012). A synthesis of remote sensing and local knowledge approaches in land degradation assessment in the bawku east district, ghana. International Journal of Applied Earth Observation and Geoinformation, 14(1), 204213.

Zhang, G. et al. (2019). Monitoring lake level changes on the Tibetan Plateau using ICESat altimetry data. Science of the Total Environment, 655, 1390–1401.