# Environment Setup

This page loads all shared dependencies, authenticates to Google Earth Engine, and derives the study area boundary and hydrological base layers used throughout the analysis.

## Packages, GEE Authentication, and AOI

All computational pages source _common.R, which handles package loading, GEE authentication, helper function definitions, and AOI derivation from HydroSHEDS level-6 basins.

source("_common.R")

## Area of Interest

We derive the AOI from HydroSHEDS basin boundaries filtered by a point location within Lake Chilwa. The level-6 basin becomes the primary clip boundary for all subsequent raster and vector queries.

print(paste("Level 6 basins containing point:", nrow(basin_l6_sf)))

[1] "Level 6 basins containing point: 1"

print(paste("Level 8 sub-catchments:", nrow(basin_l8_sf)))

[1] "Level 8 sub-catchments: 1"

tmap_mode("view")
tm_shape(st_as_sf(aoi_sf)) + tm_borders(col = "red", lwd = 2) +
  tm_shape(basin_l8_sf) + tm_borders(col = "blue", lwd = 0.5, lty = "dashed") +
  tm_shape(st_as_sf(poi_sf)) + tm_symbols(fill = "green", size = 1) +
  tm_basemap("Esri.WorldImagery") +
  tm_scalebar(position = c("right", "bottom"))

## Hydrological Base Layers

We import elevation and processed hydrology data from multiple sources for cartographic and cross-validation purposes: MERIT Hydro (v1.0.3), HydroSHEDS (v1.1), and JRC Global Surface Water (v1.4).

# HydroSHEDS: conditioned DEM (~90m)
dem              <- ee$Image("WWF/HydroSHEDS/03VFDEM")$clip(aoi_ee)
hydro_flowdir    <- ee$Image("WWF/HydroSHEDS/03DIR")$clip(aoi_ee)
hydro_flowacc    <- ee$Image("WWF/HydroSHEDS/03ACC")$clip(aoi_ee)
hydro_rivers     <- ee$FeatureCollection("WWF/HydroSHEDS/v1/FreeFlowingRivers")$filterBounds(aoi_ee)
hydro_lakes      <- ee$FeatureCollection("projects/sat-io/open-datasets/HydroLakes/lake_poly_v10")$filterBounds(aoi_ee)
hydro_rivers_sf  <- ee_to_sf(hydro_rivers)
hydro_lakes_sf   <- ee_to_sf(hydro_lakes)

# MERIT Hydro: flow direction, upstream drainage, water elevation, channel width
merit_flowdir    <- ee$Image("MERIT/Hydro/v1_0_1")$select("dir")$clip(aoi_ee)
merit_drainup    <- ee$Image("MERIT/Hydro/v1_0_1")$select("upa")$clip(aoi_ee)
merit_elev       <- ee$Image("MERIT/Hydro/v1_0_1")$select("elv")$clip(aoi_ee)
merit_wth        <- ee$Image("MERIT/Hydro/v1_0_1")$select("wth")$clip(aoi_ee)
merit_rivers     <- merit_drainup$gt(10)

# JRC Global Surface Water
gsw_occurrence   <- ee$Image("JRC/GSW1_4/GlobalSurfaceWater")$select("occurrence")$clip(aoi_ee)
gsw_seasonality  <- ee$Image("JRC/GSW1_4/GlobalSurfaceWater")$select("seasonality")$clip(aoi_ee)
gsw_recurrence   <- ee$Image("JRC/GSW1_4/GlobalSurfaceWater")$select("recurrence")$clip(aoi_ee)
gsw_transition   <- ee$Image("JRC/GSW1_4/GlobalSurfaceWater")$select("transition")$clip(aoi_ee)
gsw_monthly      <- ee$ImageCollection("JRC/GSW1_4/MonthlyHistory")$filterBounds(aoi_ee)

print(paste("JRC monthly images:", gsw_monthly$size()$getInfo()))

[1] "JRC monthly images: 454"

print(paste("Number of lakes within watershed:", nrow(hydro_lakes_sf)))

[1] "Number of lakes within watershed: 3"

tmap_mode("view")
tm_shape(st_as_sf(aoi_sf)) + tm_borders(col = "red", lwd = 2) +
  tm_shape(hydro_rivers_sf) + tm_lines(col = "steelblue", lwd = 1.5) +
  tm_shape(hydro_lakes_sf) + tm_polygons(fill = "steelblue", lwd = 1) +
  tm_shape(st_as_sf(poi_sf)) + tm_symbols(fill = "green", size = 1) +
  tm_scalebar(position = c("right", "bottom")) +
  tm_basemap("Esri.WorldImagery")